FAQs

Cannon AFB PFOS/PFOA

Actions Taken

Q. What actions has the Air Force taken at Cannon AFB about this issue?

A. The US Air Force completed a preliminary assessment and issued a report in October, 2015 identifying locations on Cannon AFB where AFFF may have been released. The site inspection was completed in August, 2018 and an expanded site inspection completed in March, 2019. The Air Force obtained 25 water samples and identified three drinking water locations where PFOS/PFOA exceeded the US Environmental Protection Agency's Lifetime Health Advisories (HAs) for PFOS/PFOA in drinking water. Two samples were below the HAs, and the remaining samples were “non-detect.”

Q. As a result of these steps, what are the actions that the Air Force has taken in response?

A. The US Air Force has provided alternate water sources in place of drinking water where PFOS/PFOA was present at levels above the US Environmental Protection Agency's (EPA’s) HAs; replaced the original AFFF mixture with a formulation that meets EPA guidelines; eliminated AFFF by incineration at approved facilities; replaced AFFF in fire vehicles; and retrofitted 850 Air Force fire vehicles with a system that prevents foam discharge in 2019. Additionally, the Air Force implemented stringent AFFF training and use guidance that limits use of AFFF to emergencies and requires immediate clean-up of any dispersal.

Q. What are the next steps that Cannon AFB is taking?

A. The US Air Force is beginning a remedial investigation at Cannon AFB to determine the nature and extent of PFOS/PFOA, which will help determine how and when we can execute any other remedial actions. The nature and extent will better define where the PFAS impact is and the concentration levels by checking surface and groundwater as well as sediment and soil. The RI contract was awarded August 31, 2020.

Q. What is a remedial investigation?

A. The RI is a very detailed investigation into PFOS and PFOA to determine if it is in surrounding soil, sediment, surface water, fish and wildlife. The investigation also collects data about site conditions, conducts risk assessments to determine whether PFOS and PFOA present unacceptable risks, and further identifies what is needed for a comprehensive cleanup at current and former installations.

Q. Why does the Air Force have to do all these studies instead of taking action?

A. The US Air Force is responding to PFOS/PFOA impacts from AFFF releases at Cannon AFB in accordance with the Comprehensive Environmental Response, Compensation, and Liabilities Act (CERCLA), the Defense Environmental Restoration Program statute (DERP), and the National Contingency Plan (NCP).  CERCLA, DERP, and the NCP are federal laws and regulations that specify a process that must be followed when responding to releases of hazardous substances, pollutants, or contaminants to the environment.  The investigations and studies the US Air Force is conducting about PFOS and PFOA released at Cannon AFB are required under the CERCLA/NCP process.  These investigations and studies are necessary to enable the Air Force to understand the nature and extent of the threat posed by PFOS/PFOA released from Cannon AFB and equip the Air Force to decide what long term steps must be taken to address that impact.

Q. How long will clean-up take in and around Cannon AFB?

A.  The CERCLA process is deliberate and driven by carefully acquired and studied data. The RI is the prerequisite to determine the nature and extent of the PFAS impact which will help decide the timeline for clean-up, and we are working as quickly as possible to perform the RI and maintain the integrity of the analysis. But it is important to note that the Air Force has taken measures to protect drinking water sources while we conduct the investigation.

Q. What is CERCLA?

A. Congress established the Comprehensive Environmental Response, Compensation, and Liability Act (CERCLA)—also known as Superfund -- in 1980 in response to risks to human health and the environment posed by the release of CERCLA-defined hazardous substances, pollutants, or contaminants to the environment. The process promotes accountability, community involvement and long-term protectiveness. The goal of CERCLA is to protect human health and the environment by cleaning up sites that present an unacceptable risk. The CERCLA process depends on regulatory standards to fully identify and resolve impacts. Without established standards, federal and state agencies are often unable to use taxpayer dollars to investigate and respond to undefined impacts, and regulatory agencies lack the ability to enforce action.

Q: Why isn’t the Air Force addressing agricultural wells?

A: Cannon AFB ranked in the top 10 of prioritized active US Air Force installations and is one of the first installations funded for the RI.  Our first step was determining if PFOS/PFOA was present in the environment, and if so, could it reach drinking water sources. The Air Force’s focus has been on addressing possible health risks to drinking water sources because drinking water is the most direct route to human consumption and because the EPA has an established Lifetime Health Advisory to help guide our actions. We immediately addressed drinking water concerns at Cannon, and have been consistently moving forward in the CERCLA process. Now we’re moving on to the RI phase, which sets the groundwork for remedial actions in the future. Those remedial actions will address risk to all receptors, to include humans, livestock, and agriculture.

Q: Isn’t the Air Force lack of action allowing the “plume” to migrate and cause more “impacts?”

A: The US Air Force is acting as quickly as possible while following the CERCLA process. The RI will determine the nature and extent or rather exactly where and how far the PFAS impact has travel so we can identify the best course of action to respond to the PFAS impacts. In the interim, we are ensuring alternative safe drinking water is available.

Q: What is the status of the ongoing litigation at Cannon AFB?

A: Litigation is ongoing and still pending resolution.

Next Steps

Q. How will the AF inform the public of efforts and results?

A. Once the RI is complete, the final report will be posted to the Administrative Record. The Air Force will also share the results with regulators for review and feedback/input.

Q. Will the AF perform interim corrective measures during investigation?

A: The US Air Force is working with a local dairy to install two Point-of-Use water treatment systems at water disbursement sites. POV is a small treatment system that is installed at the disbursement of drinking water sources.

Q. Will the Air Force conduct sampling of human serum and cattle to determine PFAS levels?

A. The primary focus is to sever pathways to protect drinking water used for human consumption. The purpose of the RI is to determine the nature and extent of the PFAS impact on drinking water sources. Addressing anything beyond drinking water requires a whole-of-government approach to set promulgated standards for any other source. For more information on human sampling, visit ATSDR’s website at https://www.atsdr.cdc.gov/

Q. How will the Air Force protect Clovis drinking water wells and other purveyors if it is determined Air Force operations are causing Lifetime HA exceedances?

A. The RI will provide data to determine the nature and extent to any off-site private or public well. If any exceedances are identified above the Lifetime HA in drinking water sources, the US Air Force will provide alternative drinking water solutions.

Q. What methodologies/technologies will be used to treat PFOS/PFOA impacted groundwater?

A.  The RI and Feasibility Study will take the Air Force one step closer to implementing long term actions and will help us select the most effective cleanup option for each installation. The Feasibility Study will evaluate the data provided by the RI and help inform the best methods and technologies for remediation.

Q: Is Cannon AFB still using PFOS/PFOA impacted AFFF for firefighting activities?

A. The US Air Force has replaced all legacy C8 AFFF at Cannon AFB with newer environmentally-compatible C6 AFFF. Additionally, the US Air Force has taken measures to prevent accidental discharge of foam from emergency vehicles, and restricted training with foam to training areas equipped to capture foam. Any C6 AFFF used for emergency situations is containerized and disposed waste in accordance with governing regulations.

Issues

Q. Will the Air Force follow State promulgated standards?

A. When a State properly promulgates a PFAS drinking water standard, the US Air Force adheres to the standard in any location where we are the supplier of drinking water. Where the Air Force is not the supplier of drinking water, we follow our authorities available in the CERCLA, which is a Federal cleanup law. Where the Air Force is a known source, CERCLA allows us to provide bottled water, filter systems, and other removal actions to quickly assure no one is drinking water above the EPA PFOS/PFOA Lifetime HAs. The Air Force is evaluating its legal authorities, and will engage with our Federal agency partners on how to address State standards in the CERCLA process.

Q. What are section 343 and 344 of the NDAA about?

A. Section 343 of the National Defense Authorization Act (NDAA) for Fiscal Year 2020 provides DoD components discretionary authority to use Operation and Maintenance funds to provide alternative water or treat DoD impacted surface or well water used for agricultural purposes to create products destined for human consumption when the water is impacted by PFOS/PFOA from activities on a military installation at levels that are above either EPA's LHAs for PFOS/PFOA or a US Food and Drug Administration standard for PFOA and PFOS in raw agricultural commodities and milk.

Section 344 authorized the acquisition of real property within the vicinity of a US Air Force base that has shown signs of PFOS/PFOA impact due to activities on base and which would both extend the contiguous geographic footprint of the base and increase the force protection standoff near critical infrastructure and runways.

Q. Will the Air Force buy dairy-owned land near the installation?

A. Section 344 of the NDAA for Fiscal Year 2020 gives the US Air Force authority to purchase land that has shown signs of PFOS/PFOA impact due to activities on base and which would both extend the contiguous geographic footprint of the base and increase the force protection standoff near critical infrastructure and runways.  Any purchase of dairy-owned land near Cannon AFB must meet the criteria specified in Section 344 and must be approved by the Assistant Secretary of Defense for Sustainment, pursuant to guidance the Air Force received from the Department of Defense about implementing Section 344.

Q. What is the US Air Force position on reimbursing agriculture industry for business losses?

A. Section 343 of the National Defense Authorization Act (NDAA) for Fiscal Year 2020 provides DoD components discretionary authority to use Operation and Maintenance funds to provide alternative water or treat DoD impacted surface or well water used for agricultural purposes to create products destined for human consumption when the water is impacted by PFOS/PFOA from activities on a military installation at levels that are above either EPA's LHAs for PFOS/PFOA or a US Food and Drug Administration standard for PFOA and PFOS in raw agricultural commodities and milk.  Neither the NDAA nor CERCLA provide any authority for the US Air Force to reimburse dairy farms for business losses due to PFOS/PFOA impact from releases of AFFF at Cannon AFB.

Q. The NDAA gives the Air Force authority to go treat right now, why isn’t the Air Force taking action?

A.  Section 343 of the National Defense Authorization Act (NDAA) for Fiscal Year 2020 provides DoD components discretionary authority to use Operation and Maintenance funds to provide alternative water or treat DoD impacted surface or well water used for agricultural purposes to create products destined for human consumption when the water is impacted by PFOS/PFOA from activities on a military installation at levels that are above either EPA's LHAs for PFOS/PFOA or a US Food and Drug Administration standard for PFOA and PFOS in raw agricultural commodities and milk.

Consistent with the its risk-based cleanup program and Department of Defense implementation guidance for Section 343, the US Air Force may exercise its Section 343 discretionary authority to provide alternative water or treat agricultural water based on meeting two criteria.  The first criterion is a scientifically supportable determination of a need to take action due to an unacceptable risk to human health or the environment based on the site-specific exposure.  Such a determination is made either after completion of risk assessments performed during the CERCLA process or by use of US Food and Drug Administration standards for PFOS/PFOA in raw agricultural commodities and milk. Since the Food and Drug Administration has not established any standards for PFOS/PFOA, the Air Force must complete risk assessments during the Remedial Investigation before it can even consider treating agricultural water using its Section 343 authority.

Q. Why has the AF not resampled to determine extent of PFOS and PFOA impact?

A. The Air Force worked as quickly as possible to award the RI to determine the nature and extent of the PFOS/PFOA impact.  The SI only validated the presence of the impact, now the RI will help the Air Force understand how far out and how big of an effect the PFOS/PFOA impact has as well as identify potential pathways and concentration levels of the impact.

Q. Why won’t the Air Force host a Restoration Advisory Board at Cannon AFB?

A. In September, 2020, the U.S. Air Force solicited public interest in a RAB. No public responses were received during the public comment period. To ensure optimal opportunity for the public, the U.S. Air Force reassessed for public interest in RAB in February 17, 2021 with a 45-day public comment period ending April 3, 2021. For more information regarding the RAB, please visit https://www.cannon.af.mil/Environmental/. The Air Force also makes information available through the Air Force Administrative Record, the Cannon AFB web site, and public meetings.

Q. I recently found out that I have cancer, is it possible that PFOS and PFOA caused my cancer?

A.  The US Air Force cannot answer your medical question.  As members of the communities where we serve, we share community concerns about human health impacts from use past use of AFFF at Cannon AFB, and we support a whole-of-government approach to resolve community concerns. For health-related concerns, we recommend you reach out to your medical provider, and local, state or federal health agencies for more information on potential PFOS/PFOA impacts.

Q. How do we know we can trust the Air Force?

A. The US Air Force is committed to transparency and ensuring the community is aware of the ongoing efforts to address the PFOS and PFOA occurrence in and around Cannon AFB. The Air Force will host frequent meetings to address public concerns and ensure that RI results and information are published on the US Air Force Civil Engineer Center CERCLA Administrative Record https://ar.afcec-cloud.af.mil/ for public review.

Q. Will the Air Force pay for independent studies and confirmation of investigation findings?

A. The US Air Force is responding to PFOS/PFOA impacts in accordance with CERCLA and the National Contingency Plan, which prescribe how the Air Force will investigate and respond to releases of hazardous substances, pollutants, and contaminants to the environment.  The Air Force can complete the CERCLA/NCP process without utilizing independent studies

Q. The New Mexico Environment Department (NMED) is also studying PFOS/PFOA occurrence in NM.  Is the Air Force partnering with NMED to ensure a robust and active approach to mitigate the PFAS impact?

A. The US Air Force shares the results of its PFOS/PFOA investigation with NMED and will continue to do so.

Q. It has been two years since the community was briefed on PFOS/PFOA impact. Why did the Air Force cease communication with the public on an issue that not only has impacted drinking water sources, but has also has tremendous economic impacts on the local dairy communities?

A.  The US Air Force has continued moving forward with our CERCLA response to PFOS/PFOA. In fact, the Air Force Civil Engineer Center (AFCEC) awarded the remedial investigation (RI) contract in August of 2020, surpassing the initial projection and commitment for award by Fiscal Year 2022. Throughout the process we have kept regulators and local elected representatives updated. Most recently, Air Force representatives met with elected officials in November, 2020. As a result of that meeting, we are hosting this public meeting, and will continue to provide information to the community and its leaders on the progress of our efforts.

Q. Will the Air Force work in collaboration with the Clean Water Act Partnership and local diary communities to ensure transparency and to share real time information?

A. The US Air Force is responding to PFOS/PFOA impacts in accordance with CERCLA and the National Contingency Plan, which prescribe how the Air Force will investigate and respond to releases of hazardous substances, pollutants, and contaminants to the environment.  Publicly releasable documents will be made available for public comment and review on the US Air Force Civil Engineer Center CERCLA Administrative Record. https://ar.afcec-cloud.af.mil/.

Q. The Air Force has stated that Department of Defense is funding cleanup technology research and development.  What is the nature of these studies and who are the key agencies?

A. The Department of Defense (DoD) has funded research on cleanup technologies that include in situ and ex situ groundwater treatment, treatment for investigative derived waste, and wastewater treatment. These technologies are at various stages of study.  The treatment technologies, investigators, and the current status can be found on the DoD PFAS page (https://serdp-estcp.org/Featured-Initiatives/Per-and-Polyfluoroalkyl-Substances-PFASs/DoD-PFAS-Page/DoD-PFAS-Page).

AFCEC seeks to advance innovative environmental technology through a Broad Agency Announcement (BAA) initiative. The BAA provide a bridge from R&D successes to use by site remediation practitioners.

Several BAA projects focusing on PFASs are underway currently at Department of the Air Force installations. Additional PFAS–related BAA projects are scheduled for award in FY21.  Current projects include:

- Wright-Patterson AFB, Ohio.  Coupling Ion-Exchange Resin with Electrochemical Treatment for Complete Separation and Destruction of PFOS and PFOA in Groundwater.

- Cape Canaveral Space Force Station, Florida.  Field-Scale Comparison of Adsorbents for In Situ Stabilization of Poly- & Perfluorinated Alkyl Substances (PFAS). A full scale application of this technology recently was initiated at Ellsworth AFB, South Dakota.

- Wright-Patterson AFB, Ohio.  Enhanced Contact Electrical Discharge Plasma Reactor (PFAS Remediation). A follow-up project seeks to upscale to water treatment rates of 10-100 gal/min.

- Peterson Space Force Base, Colorado.  Electrochemical treatment by nanofiltration plus sequential UV oxidative/reductive treatment of reject water.